# Protectoseal Series 830 vs 830E Conservation Vent Comparison

Both the Protectoseal 830 and 830E Series are end-of-line combination conservation vents and flame arresters that provide pressure and vacuum relief for tank vapor spaces while preventing external ignition. They both feature weight-loaded pallets, FEP film seating for low leakage, and air-cushioned sealing surfaces. The key distinction lies in certification standards, available sizes, and targeted markets.

| Feature | Series 830 | Series 830E |
| --- | --- | --- |
| Certifications / Standards | FM approved (2", 3", 4"), UL listed (6", 8", 10"). | ATEX / CE / ISO 16852 compliant for flame arresters. |
| Explosion Group Coverage | NEC Group D / IEC Group IIA vapors (unconfined deflagration). | Group IIA / D vapors, with full ATEX / CE certification. |
| Size Range | 2" (DN 50) to 10" (DN 250). | 2" (DN 50) to 6" (DN 150). |
| Pressure / Vacuum Settings | Typical: 0.5 to ~16 oz/in² (varies by size). | Typical: 0.5 to ~13.8 oz/in² pressure, ~8.5 oz/in² vacuum. |
| Leakage Specification | ≤1 SCFH at 90% of set pressure; FM/UL listed performance. | ≤1 SCFH at 90% of set pressure; ATEX/CE certified performance. |
| Design Focus | Broader size range, FM/UL markets, traditional US installations. | Regulated or European applications requiring ATEX/CE compliance. |

In summary, both models perform the same venting and flame-arresting function. Select the 830 for FM/UL-compliant installations or when larger sizes (up to 10") are required. Choose the 830E when ATEX/CE/ISO certification or European compliance is mandated.

Created by The Collins Companies as an easy side-by-side comparison. Please consult a sales representative before ordering to ensure accuracy for your application .